### STEM Challenge

### Resisting Destruction from Earthquakes

​

Resource Sections:

- Engineering Design Process Explained

- Earthquake questions and answer key

- Instructions for teachers

- Materials

- Lesson Plans

- Grading rubric

The Engineering Design Process Phases

- Define the Real-World Context: Anchor the challenge in the real-world problem of earthquake destruction and the role of geophysicists (scientists who study Earth's physical processes, such as earthquakes).

- Identify the Design Problem: Define the goal (tallest structure) and the constraints (materials and "shake" survival; survival means that the structure does not collapse when shaken during the simulated earthquake).

- Brainstorm & Design: Use background knowledge on seismic waves and building foundations to sketch initial ideas.

- Build—Test—Evaluate: Construct the model, test it on a shake table for 10 seconds, and collect data for improvements.

- Share & Reflect: Present designs, justify them with data, and discuss applied engineering considerations.

Earthquake Science Questions & Answer Key

Watch a short video answering these questions as a class.

​

Questions:

- How are earthquakes created?

- Where do most earthquakes occur in the world?

- Identify the causes of destruction from earthquakes.

- Research one historical Canadian earthquake. When did it occur, and what was its impact?

Answer Key:

- Answer: Earthquakes occur when underground rock breaks and moves rapidly along a fault surface.

- Answer: Most earthquakes happen along plate boundaries, especially in the "Ring of Fire" around the Pacific.

- Answer: An earthquake releases a large amount of energy quickly in the form of waves. These waves cause the Earth’s Crust to shake in all directions at different frequencies.

- Answer: (Example) The 1700 Cascadia earthquake off British Columbia was magnitude 9.0 and caused a massive tsunami.

​Instructions for Teachers:

- Material Management: Use the honour system or pre-count marshmallows/Berries. Let students pick their linguini.

- Modelling the "Shake": Simulate a moderate earthquake, rated 5 on the Moment Magnitude Scale. Ensure the container stays on the table.

- Safety Inspector: Only shake under supervision for official data.

- Foundations: If asked, allow taping the building to the base—this model's real-life engineering.

Materials:

To complete the earthquake-resistant building challenge, students will need specific materials for both the structures themselves and the DIY shake table used to test them.

​

Building Materials (Per Student Group)

- 10 pieces of linguini

- 20 mini marshmallows or Swedish berries

- 30 centimeters of tape

Tools (Per Student Group)

- Scissors and ruler

Shake Table Assembly Materials (Per Student Group)

- Two hardcover textbooks

- Four large rubber bands (7-inch size)

- Four marbles or rubber balls

​

Lesson Plans:

​

Version 1: Extended Challenge (Two 60-Minute Classes)

Class 1: Background & Design

- Context & Problem (20 mins): Introduce earthquake concepts (see Questions and Answers) and challenge constraints.

- Shake Table Assembly (20 mins): Students build shake tables using two hardcover books. They use four marbles or rubber balls, rubber bands, and blocks or cubes to simulate potential damage to infrastructure from earthquakes. Provide video instructions.

- Brainstorming (20 mins): Students sketch "Possible Solutions" individually for 3 minutes, then collaborate to select a final design.

Class 2: Building & Evaluation

- Construction (30 mins): Teams build their structures using 10 pieces of linguini, 20 marshmallows or Swedish berries, and 30 cm of tape.

- Testing (20 mins): Official height measurements are taken. Students simulate an earthquake on their shake table by having two students shake at different speeds in different directions for 10 seconds. Both students are shaking the structure simultaneously.

- Reflection (10 mins): Groups discuss successes, challenges, and factors beyond seismic concerns, such as the area's geology and geographic location.

Version 2: One 45-Minute Class

- Introduction & Context (10 mins): Briefly define the problem and constraints. The teacher should provide pre-assembled shake tables to save time.

- Rapid Brainstorming (5 mins): Groups quickly sketch one design.

- Construction (20 mins): Intensive building session.

- Testing & Gallery Walk (10 mins): Students test their buildings in a "lightning round" while others observe the results.

Grading Rubric:			Name (s):_________________________________________________

| Criteria | 4 - Exceptional | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Constraints | Used exactly 10 spaghetti, 20 marshmallows, and 30cm of tape. | Minor deviation in material use. | Significant deviation in material use. | Did not follow material constraints. |
| Stability | Structure remained completely intact for 10 seconds. | The structure swayed but did not fall over. | Structure partially collapsed. | The structure fell over immediately. |
| Height | Tallest structure in the class (antenna not included). | Above-average height. | Average height. | Below-average height. |
| Process | Clear evidence of brainstorming and reflection. | Completed design and testing phases. | Rushed through the design phase. | No evidence of planning. |
| Attitude | Students worked together collaboratively. All students’ ideas were heard, and everyone stayed positive. | Students worked together collaboratively. Most students’ ideas were heard, and most everyone stayed positive. | Students did not work together collaboratively or communicate efficiently. The group was frustrated at times. | Students did not work together or communicate. The group was frustrated and gave up on their structure. |
| Feedback: ​ |  |  |  | /20 ​ = % |

​